# HDMI Spec Notes

## Signal Information

Describing the source of signals and basic information about their functions

- +5V sent from the source whenever it’s using the DDS or TMDS signals

- DDS Channel (I2C Channel, meeting specs version 2.1 section 15 for standard mode):

  - SDA, SCL, DDC/CEC GROUND

- CEC Line:

## Operating Modes

- Video Data Period

  - Pixels of an active video line are transmitted

  - This means a literal horizontal line on the monitor

  - TMDS for 8 bits per channel per clock period, 24 bits total per pixel

- Data Island Period

  - Audio and auxiliary data transmitted as a series of packets

  - 4 bits per channel per clock period, 12 bits total

- Control Period

  - No video, audio, or auxiliary data needs to be transmitted

  - Required between any two non-control periods

  - Transition maximized encoding

## Control Period

Transmits the Preamble and allows the sink to perform character synchronization

There are two preamble encodings:

- Video Data Period: CTRL(0:3) = 1000

- Data Island Period: CTRL(0:3) = 1010

Synchronization:

- The control period uses transition-maximized encodings, which makes it useful for the sink to synchronize

- Necessary because the pixel clock DOES NOT exactly match the speed of transmissions. Different modes could run at 2x, 4x, etc. Need a counter for synchronization to know how to line up with the pixel clock

## Video Data Period

Two character Video Leading guard band with predefined values. No trailing guard band. After leading, 24 bits of pixel information are transmitted every tmds clock period.

# Repeater

A Repeater is defined as a device with one or more HDMI inputs, one or more HDMI outputs, and a retransmission function.

- This is what I’m making

- Each input device shall fulfill all requirements of an HDMI sink, each output all requirements of a source